# EOS Utility 3.20 Watch Lab Cheat Sheet

Validated with EOS Utility 3.20.21.1 + EOS R10

# Remote Live View — what matters

| Control | Meaning |
| --- | --- |
| Lens-distance button | Show/hide focus distance and magnification scale. |
| Electronic level | Animated pitch/roll overlay; useful for setup, distracting for shooting. |
| Auto Rotate | Automatic Live View rotation. |
| Manual rotate | Rotate display when Auto Rotate is off. |
| AF Point Disp | Show/hide the inner AF square. Must be visible for the lab’s double-click AF workflow. |
| Aspect ratio | Aspect-ratio / crop information controls. |
| Outer-frame toggle | Show/hide the magnification frame. |
| ×1 / ×5 / ×10 | Normal, inspection, and critical-focus views. |

# The double-click focus method

| Action | Result |
| --- | --- |
| Drag inner AF point | Moves the AF target. |
| Double-click inside inner AF point | Runs AF; successful One-Shot focus turns the point green. |
| Double-click somewhere else | Moves the magnification/AF location there and opens 5×; does not yet run AF. |
| Double-click the relocated AF point | Runs AF at that new location. |
| Double-click with AF Point Disp off | Magnifies rather than starting AF. |
| Outer frame only, no inner AF point | Double-click magnifies; outer frame is not an AF point. |

# Fine focus

| Amount | Closer | Farther |
| --- | --- | --- |
| Large | I | O |
| Medium | K | L |
| Small | < / comma | > / period |

LAB SHORTCUT: On this R10/EOS Utility 3.20 setup, the small electronic focus nudges work while the capture window remains in AF. That makes the fastest workflow: AF → 10× → tiny nudge → shoot. Canon’s documented manual-focus procedure uses the capture-window MF setting; both approaches are valid when carefully controlled.

# Single-shot sequence

STEP  Remote Live View → compose

STEP  AF Point Disp ON

STEP  Move AF point

STEP  Double-click point → green

STEP  10× → verify / nudge

STEP  1× → remote release

STEP  Inspect CR3 at 100%

# Focus-stack sequence

STEP  Place AF point on nearest plane that must be sharp

STEP  AF → 10× verify

STEP  Focus Bracketing: 30 shots / increment 3 / smoothing ON / Depth Composite ON

STEP  Trigger once

STEP  Do not touch rig until sequence completes

STEP  Inspect composite plus first/middle/last source frames