# R10 Watch Lab Gold Configuration

C1 macro preset + C2 RF28mm walk-around preset

C1 MISSION: Tripod-mounted RF100mm watch photography controlled primarily from EOS Utility.

# C1 — Watch Lab

| Setting | Value |
| --- | --- |
| Mode | M |
| ISO | 100 |
| Aperture | f/8 |
| Shutter | As required by lighting |
| Image quality | RAW (.CR3) |
| White balance | Fixed / repeatable |
| AF operation | One-Shot |
| AF area | 1-point AF |
| Subject tracking | Off |
| Subject to detect | None |
| Eye detection | Disable |
| Preview AF | Off |
| Lens operation if AF unavailable | Continue focus search |
| Shutter half-press | Metering only |
| AF-ON | Metering + AF start |
| Shutter mode | Electronic 1st-curtain |
| Drive | Single |
| Focus bracketing | Enable when stacking |
| Exposure smoothing | On |
| Depth composite | On while learning |
| Crop depth composite | On |
| HDR mode / HDR PQ | Off / Off |
| Lens AF/MF | AF |
| Focus limiter | FULL |
| IS | Off |
| SA ring | Neutral + locked |

# C2 — RF28mm Walk-Around

| Setting | Value |
| --- | --- |
| Mode | Av |
| Aperture | f/4 starting point |
| ISO | Auto, practical ceiling 6400 |
| Minimum shutter | 1/250 s starting point |
| AF operation | Servo |
| AF area | Whole area AF |
| Subject | People |
| Eye detection | Enable / Auto |
| Tracking | On |
| Drive | High-speed continuous |
| Image quality | RAW |
| Back-button AF | Same mapping as C1 |
| Auto update | Disable |

# Save / refresh C1 or C2

STEP  Set the camera exactly as desired in M or Av.

STEP  MENU → Set-up → Custom shooting mode (C1, C2).

STEP  Register settings → C1 or C2 → OK.

STEP  Set Auto update set. = Disable.