# Watch Lab Shooting Checklist

Overhead RF100mm + EOS Utility

# 1. Camera & Lens

☐ C1 selected

☐ Battery / USB power ready

☐ SD card inserted with space available

☐ RF100mm switch = AF

☐ Focus limiter = FULL

☐ IS = OFF

☐ SA ring = neutral and locked

☐ Tripod rigid; camera angle confirmed

# 2. Watch & Light

☐ Crystal, case, bracelet, background cleaned

☐ Watch orientation and hand position set

☐ Diffusion in place

☐ Reflectors / flags positioned

☐ Lights warmed up and stable

# 3. EOS Utility

☐ USB connected; Remote Shooting open

☐ Remote Live View open

☐ Destination verified: PC + card

☐ RAW selected

☐ M / ISO 100 / f/8

☐ Fixed white balance

☐ Preview AF OFF

☐ 1-point AF; tracking OFF; subject None; eye OFF

☐ Histogram checked; polished highlights controlled

# 4. Focus

☐ AF Point Disp ON

☐ Place AF point on a high-contrast feature

☐ Double-click AF point; confirm green

☐ Inspect at 10×

☐ Use tiny electronic focus nudges if needed

# 5A. Single Shot

☐ Return to 1×

☐ Trigger remotely

☐ Inspect image at 100%

☐ Check dust, reflections, highlight clipping, dial text, markers

# 5B. Focus Stack

☐ Choose nearest required sharp plane

☐ Focus Bracketing enabled

☐ 30 shots / increment 3 starting point

☐ Exposure smoothing ON

☐ Depth Composite ON while learning

☐ Crop depth composite ON

☐ Trigger once; touch nothing until complete

☐ Check first / middle / last source frame

☐ Check composite for gaps or artifacts

# 6. Session Close

☐ Confirm files are on PC

☐ Confirm backup on SD card

☐ Flag keepers for Luminar Neo

☐ Power down lights

☐ Disconnect USB after EOS Utility closes